# NGU – School of Engineering (SOE)

Computer, Communication and Autonomous Systems (CCAS)

CCAS.5.1 – Graduation Project

Smart Sign Language Recognition Glove

Using Sensor Data and Transfer Learning

Malak Hesham - 202200011

Malak Ehab - 202200069

Zeina Shebl - 202200080

Omar Atef - 202200350

Taha Sherif - 202202655

Omar Hassan - 202202752

# Abstract

For millions of deaf and mute people throughout the world, sign language serves as their major means of communication.  However, communication barriers still persist due to limited understanding of sign language among the general population. In this project, we will present the design and implementation of a sensor-based smart glove for recognizing American Sign Language (ASL) alphabets and translating them into text and speech.

Unlike vision-based approaches which suffer from lighting, background, and privacy limitations, the proposed system uses wearable sensors to directly capture both finger bending and hand posture. To address this challenge of limited labeled sensor data, a transfer learning approach will be adopted. Hand gesture image datasets are first processed using a hand landmark detection model, and the extracted joint coordinates are converted into vector representations that imitate the sensor readings. A machine learning model is initially trained on this synthetic dataset and it is  then fine-tuned using real sensor data collected from the glove.

The system combines embedded hardware, machine learning, and assistive technology to provide a low-cost, portable, and a reliable sign language translation solution.

# Table of Contents

Contents

Introduction	1

Problem Statement	3

Proposed System and Methodology	4

Literature Review	5

System Architecture	7

Project Phases	8

Expected Outcomes	8

Significance and Impact	8

Team Roles and Responsibilities - Zabato el roles	9

Conclusion	12

References	13

# Introduction

Assistive tech has exploded in recent years, thanks to cheaper embedded electronics and smarter AI reshaping how we interact with computers. Wearables for sign language recognition stand out because they could break down huge communication barriers for people with hearing or speech impairments. Sign languages like ASL mix finger bends, hand tilts, movements through space, and timing in super complex ways—making automated detection a tough puzzle that pulls in machine learning, computer vision, and hardware engineering. We've made real progress, but there's still a big gap between lab prototypes and affordable, everyday wearables that translate signs reliably in real time.

Most sign language research sticks to camera-based systems using deep learning like CNNs and recurrent networks to decode video gestures. They work great in labs with big vocabularies, but flop with bad lighting, messy backgrounds, occlusions, or privacy issues from always-on cameras (Koller et al., 2015). Wearable gloves flip that script by using sensors to directly track finger flexes and hand moves—no cameras needed, so they're portable and tough against the environment. Early ones like SignSpeak paired flex sensors and accelerometers with basic stats classifiers and got decent results, but they didn't scale well or handle diverse data (Bukhari et al., 2015). Newer smart gloves go wireless or flexible, but too many lean on rigid rules or tiny datasets that don't work across users (Abougarair & Arebi, 2022).

The biggest headache? We lack massive labeled datasets for sensor training—nothing like the huge video libraries out there. Collecting glove data is a hardware nightmare. So, our project tackles this with a hybrid transfer learning setup that links image data to sensor world. We grab public ASL image datasets of hand poses for letters, run them through MediaPipe to pull 21 joint coordinates per image, then convert those into normalized vectors mimicking flex angles and IMU readings. Boom—synthetic sensor data at scale for pre-training our classifier.

On the ML side, we vectorize the landmarks, train on that for broad hand gesture smarts, then fine-tune with real glove data from flex sensors and an IMU. This lets us tap image abundance while tuning to actual hardware quirks—proven to boost results without endless data hunts (Tian et al., 2020).

Beyond recognition, we've built in multilingual support for real life. ASL signs turn to English text, then pipe into other languages via translation APIs. Imagine signing ASL in Egypt: glove catches it, spits Arabic text, and a TTS voice reads it aloud. Separating recognition from translation means easy scaling—no model retrains needed. Real-time speech makes chatting with non-signers seamless.

Our Smart Sign Language Recognition Glove ties it all together: flex sensors and IMU on fingers/hand feed a low-power microcontroller; software handles vector math, AI inference, and voice output. It's cheap, portable, and fixes lit gaps like data shortages, environment woes, and language limits. Blending transfer learning, wearables, and translation, this pushes accessible tech forward—ready for daily use.

Eh el technologies that we used

Eh tafaseel el el project el ML models bte3mel eh

El translation hate3emel ezay (proxy)

ADD BOM ATEF MADE BAS MSH HENA

# Problem Statement

Despite technological advances, communication barriers persist due to limited accessibility solutions. Existing systems exhibit key limitations:

• Vision-based approaches require high computational resources
• Environmental variability reduces reliability
• Commercial data gloves are expensive
• Low-cost gloves lack large labeled datasets
• Most systems do not generalize across different users

The primary research problem addressed in this project is:

How can a low-cost wearable glove achieve robust ASL alphabet recognition despite limited labeled sensor data?

# Proposed System and Methodology

The proposed system consists of:

• A wearable sensor glove
• A machine learning classification model
• A real-time speech output module

To overcome data scarcity:

- Existing ASL image datasets are selected.

- MediaPipe extracts 21 hand landmark points.

- Joint coordinates are converted into normalized relative vectors.

- Vectors simulate flex sensor + orientation outputs.

- A classification model is pre-trained on this synthetic dataset.

- The model is fine-tuned using real sensor readings.

- The optimized model is deployed on embedded hardware.

This cross-domain transfer learning strategy distinguishes the project from traditional glove-based systems.

# Literature Review

The evolution of sign language translation gloves shows real interdisciplinary progress in wearable electronics, human-computer interaction, and machine learning.

Early glove-based systems leaned on flex sensors paired with statistical classifiers. Bukhari et al. (2015) brought out SignSpeak, which mixed in flex sensors and accelerometers. They cut down features using Principal Component Analysis, and classified with Euclidean distance. The system nailed 92% accuracy in controlled tests, but manual calibration and small datasets held back its scalability and adaptability.

Later efforts ramped up sensor use. Abougarair and Arebi (2022) created a glove with five flex sensors and accelerometers for motion tracking. Wireless comms made translation mobile. They skipped ML training models entirely. Instead, the paper details rule-based thresholding—a bunch of if(...) { letter='X'; } conditions—to map sensor angles and states to letters. It showed low translation latency, but they tested on limited samples, which raises doubts about how well it works independently of the signer.

Thanks to advances in material science, we got high-performance wearables. The UCLA glove (Tian et al., 2020) packed stretchable sensor arrays and flexible electronics. Their setup hit recognition rates up to 98.63% on hundreds of ASL signs. A key win: it captured non-manual features with facial sensors, underscoring ASL's multimodal nature. That said, those fancy sensor designs jack up costs and make fabrication way more complex.

O’Connor et al. (2017) offered a low-cost twist, zeroing in on alphabet recognition. Their binary finger-state encoding kept hardware simple, but it boxed in the expressive vocabulary.

Running alongside glove work, vision-based systems posted big wins with deep learning. Koller et al. (2015) showed large-vocabulary continuous sign language recognition using CNN-HMM hybrids. Still, these demand huge video datasets and serious computational muscle.

Across the literature, the main limitations boil down to:

- Dataset scarcity for wearable systems

- Limited cross-domain training strategies

- Lack of scalable low-cost architectures

Very few studies dig into using abundant image-based datasets to boost sensor-based systems. This project steps in to fill that gap with a transfer learning pipeline that bridges image-derived landmarks and wearable sensor data.

While previous studies have made solid progress in both wearable glove systems and vision-based sign language recognition, key technical hurdles still linger. Current sensor gloves often stick to small datasets or rule-based logic, which cramps scalability and generalization across users (Abougarair & Arebi, 2022). On the flip side, vision-based systems pull off strong results thanks to large annotated datasets, but they guzzle high computational resources and need controlled environments (Koller et al., 2015). The literature makes it clear: hardly any work tries to connect the wealth of image-based sign language data with the real-world perks of low-cost wearable sensing.

To tackle this, our project rolls out a cross-domain transfer learning framework blending computer vision landmark extraction with wearable sensor modeling. Unlike old-school glove approaches that rely solely on physical sensor recordings, we kick off with publicly available ASL alphabet image datasets as the main training source. We apply MediaPipe hand landmark detection to pull 21 joint coordinates that match finger articulation and palm orientation. Then, we transform those coordinates into normalized relative vectors using geometric normalization and scaling. These vectors mimic flex sensor readings and inertial measurements, basically simulating wearable sensor outputs straight from image data.

That generated vector dataset becomes the synthetic pre-training ground for the machine learning classifier. We train the model on a massive batch of landmark-derived vectors, so it picks up generalized spatial reps of ASL alphabet gestures before touching real hardware data. Next, we fine-tune it with actual sensor signals from the wearable glove prototype—outfitted with flex sensors and an IMU. This transfer learning approach lets the classifier handle real-world signal noise and user-to-user differences, all while holding onto the strong gesture knowledge from the bigger image dataset. It's designed to smash one of the biggest issues in prior wearables: no scalable training data.

From a system architecture angle, the proposed glove pulls together embedded sensing, machine learning inference, and real-time audio output. The microcontroller captures sensor data on finger bending and hand orientation, processes it into classification-ready feature vectors. The trained model handles alphabet recognition, spitting out textual output for recognized ASL gestures. Going beyond basics, we add a multilingual translation pipeline to cross linguistic barriers. Recognized ASL letters or words first map to English text, then translate to other languages via a proxy stage. Take a user signing ASL on a trip to Egypt—their gestures turn into Arabic speech through a text-to-speech module, making interaction with non-signers totally seamless.

The methodological heart of this work is weaving together three usually separate worlds: image-based landmark modeling, wearable sensor recognition, and multilingual assistive communication. By using vectorized landmark data as the bridge from vision datasets to embedded sensing, we craft a scalable training strategy that cuts reliance on massive hardware data collection—while delivering real-time performance fit for embedded use. This hybrid setup makes our system a practical, cost-effective step up from pure rule-based gloves and compute-heavy vision-only ones, directly tackling the core limitations from the literature.

Zawed what we will do exactly

# System Architecture

Hardware

• Arduino MKR WiFi 1010
• 5 Flex Sensors
• IMU (LSM6DS3 / BMI160)
• MAX98357A I2S DAC
• Mini Speaker
• Li-Po Battery

All components operate at 3.3V logic.

Software

• Landmark extraction (MediaPipe)
• Feature vector normalization
• Transfer learning classification
• Embedded inference optimization
• Text-to-speech output

# Project Phases

Phase 1 – Literature Review and System Design
Phase 2 – Dataset Processing and Model Training
Phase 3 – Embedded Deployment
Phase 4 – Testing and Validation
Phase 5 – Fine-Tuning and Calibration
Phase 6 – Documentation and Presentation

# Expected Outcomes

• Functional ASL alphabet recognition glove
• Real-time speech output
• Reduced sensor dataset dependency
• Scalable embedded AI architecture

# Significance and Impact

The project integrates embedded systems and artificial intelligence into a socially impactful assistive technology. By introducing cross-domain transfer learning, it addresses a core limitation in wearable sign language research.

The system demonstrates potential for startup development and broader accessibility deployment.

# Team Roles and Responsibilities

The successful development of the Smart Sign Language Recognition Glove required coordinated contributions across hardware design, software development, data processing, and system integration. The responsibilities were distributed among team members as follows:

1. Hardware & Sensor System

Owner: Omar Atef

- Glove assembly and sensor placement

- Flex sensor and IMU wiring

- Voltage divider circuit design

- Hardware calibration and stability testing

2. Embedded Firmware & Data Acquisition

Owner: Malak Ehab

- Arduino MKR WiFi 1010 configuration

- Sensor reading and data acquisition pipeline

- IMU I2C communication

- Real-time data transmission and filtering

3. Dataset Processing & Landmark Vectorization

Owner: Zeina Shebl

- ASL image dataset preparation

- MediaPipe hand landmark extraction

- Conversion of landmarks to normalized vectors

- Synthetic dataset generation

4. Machine Learning Model Development

Owner: Malak Hesham

- Classification model design

- Training on vectorized dataset

- Hyperparameter tuning

- Model evaluation and optimization

5. Transfer Learning & Model Optimization

Owner: Taha Sherif

- Cross-domain transfer learning pipeline

- Mapping landmark vectors to sensor signals

- Fine-tuning using real glove data

- Generalization testing

6. Speech Output & Multilingual Translation

Owner: Malak Ehab
 Support: Taha Sherif

- Text-to-speech integration

- Audio module implementation

- Translation proxy (English → Arabic/other languages)

7. System Integration, Testing & Documentation

Owner: Omar Hassan

- Full system integration

- Performance and latency testing

- User testing and validation

- Technical documentation and presentation

Collaborative Contributions

All team members contributes to:

- Literature review and research discussions

- System architecture design decisions

- Problem analysis and methodology refinement

- Final report writing and revision

# Timeline:

# 2-Month Development Timeline:

### Week 1 – Hardware Prototype

# Select sensors (flex sensors / IMU / accelerometer).

# Connect sensors to microcontroller (e.g., ESP32 / Arduino).

# Test sensor readings.

# Send raw sensor data to a laptop for verification.

# Basic circuit prototype on breadboard.

# Deliverable: Working glove prototype sending sensor data.

### Week 2 – Data Collection System

# Write code to log sensor data.

# Create dataset structure (gesture label + sensor readings).

# Record initial dataset for core signs.

# Generate synthetic data using augmentation techniques.

# Deliverable: First dataset version.

### Week 2 – Initial Model Training

# Preprocess dataset (normalization, filtering).

# Train first ML model.

# Test multiple architectures:

# LSTM

# CNN

# Transformer / Transfer learning

# Select the best architecture.

# Deliverable: Baseline model + architecture decision.

### Week 4 – Model Optimization

# Improve dataset quality.

# Hyperparameter tuning.

# Evaluate using:

# Accuracy

# Precision

# Recall

# F1 score

# Reduce latency for real-time prediction.

# Deliverable: Stable model ready for integration.

Add el words el hanst5dmha

### Week 5 – Final Hardware Build

# Move prototype to final glove design.

# Secure sensors in glove.

# Optimize wiring.

# Add WiFi/Bluetooth communication.

# Deliverable: Fully wearable smart glove.

### Week 6 – Mobile App Initial Version

# Build basic mobile app:

# Connect to glove via WiFi/Bluetooth

# Receive sensor data

# Integrate ML model inference.

# Display predicted sign.

# Deliverable: First working app prototype.

### Week 7 – Arabic Sign Language Dataset

# Collect Arabic Sign Language gestures.

# Record multiple users performing gestures.

# Label dataset.

# Add dataset to training pipeline.

# Deliverable: Arabic dataset added.

### Week 8 – Arabic Support + Final App

# Retrain model with Arabic gestures.

# Update mobile app:

# Show translated text

# Improve UI

# End-to-end testing:

# Glove → model → app output.

# Deliverable: Full working system.

# 1-Month Improvement Phase:

### Week 9 – Model Improvement

# Increase dataset size.

# Improve accuracy.

# Reduce false predictions.

### Week 10 – Hardware Optimization

# Improve glove comfort.

# Reduce sensor noise.

# Optimize battery usage.

### Week 11 – App Enhancement

# Better UI/UX.

# Add features:

# Speech output

# Sentence formation

# Gesture history.

### Week 12 – Final Testing & Documentation

# Full system testing.

# Performance evaluation.

# Prepare:

# Demo

# Report

# Presentation.

# Conclusion

This project walks through the design and build of a smart sign language recognition glove that pulls together wearable sensors, machine learning, and real-time speech output to make assistive communication actually work. We tackled the big headaches from existing research—like tiny wearable datasets, over-reliance on clunky rule-based methods, and no good scalable low-cost options for real-time sign language translation. By hooking up flex sensors and an inertial measurement unit (IMU) to an embedded microcontroller, the glove grabs data on finger bends and hand movements, then turns those raw signals into gesture patterns that make sense.

A huge part of this was our transfer learning strategy to beat the data crunch. Instead of scraping by with small sensor datasets, we grabbed ASL image datasets, ran them through hand landmark detection to pull joint coordinates, and morphed those into normalized vectors that act just like wearable sensor readings. That lets the machine learning model soak up gesture patterns from a massive dataset first, then fine-tune on real glove data. Result? Tougher, more reliable models without chasing down tons of hardware recordings.

We also baked in a multilingual communication pipeline built for everyday real-world chaos. Recognized ASL gestures flip to English text, then jump to languages like Arabic via a quick proxy translation, and boom—text-to-speech audio. This shows how wearable assistive devices can let users chat across languages and spots, pushing the tech way past stuffy lab benches into actual life.

Right now, we're dialed in on ASL alphabet recognition, but this sets up a rock-solid base to grow into word-level stuff and full continuous sign language translation. Next steps could mean grabbing bigger multi-user datasets, sharpening model generalization, and weaving in smarter contextual language smarts. All in all, this glove proves a hands-on, scalable way to mash embedded systems and artificial intelligence for game-changing assistive comms tech.

# References

Abougarair, A., & Arebi, K. (2022). Smart glove for sign language translation. ResearchGate. https://www.researchgate.net/publication/366741018_Smart_glove_for_sign_language_translation

Bukhari, J. F., Rehman, M., & Malik, S. (2015). American sign language translation through sensory glove; SignSpeak. International Journal of u- and e-Service, Science and Technology, 8(1), 131–142.

Koller, O., Forster, J., & Ney, H. (2015). Continuous sign language recognition: Towards large vocabulary statistical recognition systems handling multiple signers. Computer Vision and Image Understanding, 141, 108–125.

O’Connor, J., et al. (2017). Low-cost smart glove translates American Sign Language alphabet and controls virtual objects. UC San Diego Today. https://today.ucsd.edu/story/low_cost_smart_glove_translates_american_sign_language_alphabet_and_control

Tian, L., et al. (2020). Wearable tech glove translates sign language into speech in real time. UCLA Newsroom. https://newsroom.ucla.edu/releases/glove-translates-sign-language-to-speech​